Example Code Introduction for 32-bit NuMicro® Family

Drive RGB LCD Panel Using EBI

grity

| Document Information |  |
| --- | --- |
| Application | This example code uses the M55M1 EBI to drive a synchronous signal RGB LCD panel. |
| BSP Version | M55M1_Series_BSP_CMSIS_V3.01.001 |
| Hardware | NuMaker-M55M1 V1.0 |

The information described in this document is the exclusive intellectual property of
 Nuvoton Technology Corporation and shall not be reproduced without permission from Nuvoton.

Nuvoton is providing this document only for reference purposes of NuMicro microcontroller and microprocessor based system design. Nuvoton assumes no responsibility for errors or omissions.

All data and specifications are subject to change without notice.

For additional information or questions, please contact: Nuvoton Technology Corporation.

www.nuvoton.com

# Overview

The commonly used LCD panel interface for microcontrollers is the MPU-type LCD. Since this type of panel has a built-in controller and display memory, its cost is higher than that of a synchronous signal LCD panel (refer to Table 1-1 Comparison Table of MPU-type and Sync-type LCD). This example code uses the M55M1 EBI to drive a synchronous signal (Sync-Type) LCD panel. It leverages the EBI interface along with the PDMA or GDMA supported by the M55M1 series to achieve self-refresh functionality.

Figure 1-1 EBI drives a synchronous signal LCD Panel

|  | MPU-Type LCD | Sync-Type LCD |
| --- | --- | --- |
| Transmission Method | EBI i80 interface or SPI for command operations and image data transmission. | Display images using RGB and synchronous signals. |
| Memory and Bandwidth Usage | Low | High |
| Applicable Scenarios | Suitable for static images or minor dynamic image updates. | Suitable for higher resolution and dynamic images. |
| Hardware Cost | Equipped with a built-in LCD controller, outputs RGB data and synchronous signals from the panel, with a higher price. | No built-in LCD controller; the host directly outputs RGB data and synchronous signals to the panel, resulting in a relatively lower cost. |

Table 1-1 Comparison Table of MPU-type and Sync-type LCD Panel

## Principle

This example code simulates a synchronous signal LCD controller using EBI and utilizes PDMA or GDMA for memory-to-memory transmission. It transfers RGB565 pixel data to the synchronous signal LCD panel through the EBI interface. The operating principle involves establishing DMA descriptors for V Lines × H Stages based on the panel layout, where:

- V (Vertical) represents the number of scan lines for Vertical Pulse Width (VPW), Vertical Back Porch (VBP), Vertical Active (VA), and Vertical Front Porch (VFP) in the LCD timing.

- H (Horizontal) represents the four stages: Horizontal Pulse Width (HPW), Horizontal Back Porch (HBP), Horizontal Active (HA), and Horizontal Front Porch (HFP).

Figure 1-2 V Lines × H Stages DMA Descriptors

As shown in Figure 1-2 V Lines × H Stages DMA Descriptors, these V Lines × H Stages DMA descriptors are linked sequentially. The NEXT LINK of the last descriptor is set to the address of the first descriptor, forming a circular descriptor set. The last descriptor enables a transfer completion interrupt, and within the Interrupt Service Routine (ISR), the starting address for HACT phase descriptor data transmission is updated. This ensures that each lines switches to the memory start address of the new image, achieving dynamic panel updates while providing an anti-tearing function.

| V \ H | HFP | HPW | HBP | HACT |
| --- | --- | --- | --- | --- |
| VFP | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Active Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy |
| VPW | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Active Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Active Set VSYNC_ACT to Active Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Active Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Active Set DEACT to Inactive Set D[15-0] to Dummy |
| VBP | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Active Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy |
| VACT | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Active Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Inactive Set D[15-0] to Dummy | Set HSYNC_ACT to Inactive Set VSYNC_ACT to Inactive Set DEACT to Active Set D[15-0] to pixel data in every Line Raising in latest descriptor. |

Table 1-2 Correspondence of LCD Timing Stages and HSYNC_ACT, VSYNC_ACT  and DEACT Pins

Due to the varying levels of support for synchronization signals across different types of panels, some panels can support DEACT-Only synchronization mode, meaning they only require the DEACT signal input and do not need HSYNC_ACT or VSYNC_ACT signal inputs. When operating such DEACT-Only Panel, the number of DMA descriptors can be significantly reduced, saving system memory space and bandwidth usage. This is illustrated in Table 1-3 Correspondence of LCD Timing Stages and DEACT-Only Pin, as well as.

| V \ H | HFP | HPW | HBP | HACT |
| --- | --- | --- | --- | --- |
| VFP | Set DEACT to Inactive Set D[15-0] to Dummy |  |  |  |
| VPW |  |  |  |  |
| VBP |  |  |  |  |
| VACT | Set DEACT to Inactive Set D[15-0] to Dummy |  |  | Set DEACT to Active Set D[15-0] to pixel data in every Line Raising in latest descriptor. |

Table 1-3 Correspondence of LCD Timing Stages and DEACT-Only Pin

As shown in Figure 1-3 EBI16-to-RGB24 Signal Pin Connection Diagram, the HSYNC_ACT, VSYNC_ACT, and DEACT synchronization signal pins in the LCD timing are simulated through the EBI's ADR1, ADR0, and ADR7 address pins. By specifying different address decoding, LCD synchronization signals are generated, and the PCLK signal is connected to the EBI-nWR pin. The EBI's 16-bit data pins D[15-0] correspond to the LCD color signals as follows:

- R[7-3] (Red), G[7-2] (Green), and B[7-3] (Blue).

- Additional R[2-0], G[1-0], and B[2-0] can be generated using the data from R[7-5], G[7-6], and B[7-5] for RGB888 color complement, enabling higher color precision.

Figure 1-3 EBI16-to-RGB24 Signal Pin Connection Diagram

This example code requires an expansion sub-board, including the NuMaker-M55M1 development board, a 4.3” WQVGA synchronous signal panel, and an EBI16–RGB24 glue board, as shown in Figure 1-4 External Hardware Modules.

Figure 1-4 External Hardware Modules

## Execution Result

This example code provides two target compilation options: WQVGA_GDMA and WQVGA_PDMA, corresponding to the use of the GDMA or PDMA controllers for the self-refresh panel functionality, as shown in Figure 1-5 Compile Option Selection and Execution. During execution, you can observe the results by connecting the NULINK2ME through the VCOM feature and using terminal software on the computer. The default communication parameters are 115200N81. As shown in Figure 1-6 Execution Screen, the terminal will display the execution results. Users can also directly observe the image switching and updating effects on the connected panel.

Figure 1-5 Compile Option Selection and Execution

Figure 1-6 Execution Screen

# Code Description

This example code executes the initialization and termination processes for each component, with the code located in main.c. The initialization and termination functions for each component are implemented in disp_sync_gdma.c, disp_sync_pdma.c, board.c, disp_example.c, and disp.h. The component implementations use the COMPONENT_EXPORT macro to register the component name (.name), initialization callback function (.initialize), and termination callback function (.finalize). These component registration details are stored in the CompInitTab section, which is then called and executed by the main program.

/**

* @brief Initializes the components.

* @param[in]  None

* @return     None

*/

static void components_initialize(void)

{

int i;

component_export_t asCompInitTbl = (component_export_t)&CompInitTab$$Base;

uint32_t u32CompInitNum = (component_export_t)&CompInitTab$$Limit - asCompInitTbl;

for (i = 0; i < u32CompInitNum; i++)

{

if (asCompInitTbl[i].initialize)

{

printf("Initial %s\n", asCompInitTbl[i].name);

asCompInitTbl[i].initialize();

}

}

}

/**

* @brief      Finalizes the components.

* @param[in]  None

* @return     None

*/

static void components_finalize(void)

{

int i;

component_export_t asCompInitTbl = (component_export_t)&CompInitTab$$Base;

uint32_t u32CompInitNum = (component_export_t)&CompInitTab$$Limit - asCompInitTbl;

for (i = 0; i < u32CompInitNum; i++)

{

if (asCompInitTbl[i].finalize)

{

printf("Finalize %s\n", asCompInitTbl[i].name);

asCompInitTbl[i].finalize();

}

}

}

int main(void)

{

components_initialize();

/* Just keep here, or put your code here. */

while (1)

{

__WFI();

}

components_finalize();

return 0;

}

## disp_sync_gdma Component Description

In the implementation of disp_sync_gdma.c, the function disp_gdma_dsc_init sequentially creates the V Lines × H Stages DMA descriptors. For each different H/V LINE Stage, it assigns the appropriate Memory-to-memory operation attributes and write addresses to output the pixel data from the VRAM buffer to the LCD panel's HSYNC_ACT, VSYNC_ACT, and DEACT signals. This process ensures that the synchronization signals and pixel data are correctly transferred to the panel.

// Function to initialize EBI sync GDMA

static int disp_ebi_sync_gdma_init(void)

{

enum dma350_lib_error_t lib_err;

/* Set the VRAM address by default. */

s_pu16BufAddr = (uint16_t*)g_au8FrameBuf;

/* Enable GDMA module clock and un-mask interrupt. */

gdma_init();

/* Initial all Lines descriptor-link. */

disp_gdma_dsc_init();

/* Link to external command */

dma350_ch_enable_linkaddr(GDMA_CH_DEV_S[1]);

dma350_ch_set_linkaddr32(GDMA_CH_DEV_S[1], (uint32_t) s_head);

dma350_ch_disable_intr(GDMA_CH_DEV_S[1], DMA350_CH_INTREN_DONE);

dma350_ch_cmd(GDMA_CH_DEV_S[1], DMA350_CH_CMD_ENABLECMD);

return 0;

}

static int disp_ebi_sync_gdma_fini(void)

{

/* Disable GDMA module clock and mask interrupt. */

gdma_fini();

return 0;

}

## disp_sync_pdma Component Description

In the implementation of disp_sync_pdma.c, the function disp_pdma_dsc_init sequentially creates the V Lines × H Stages DMA descriptors. For each different H/V LINE Stage, it assigns the appropriate Memory-to-memory operation attributes and write addresses to output the pixel data from the VRAM buffer to the LCD panel. This process ensures that the HSYNC_ACT, VSYNC_ACT, and DEACT signals, along with the corresponding timing sequences, are correctly transferred to the panel, enabling proper image output.

// Function to initialize the EBI sync PDMA

static int disp_sync_pdma_init(void)

{

struct nu_pdma_chn_cb sChnCB;

/* Set the VRAM address by default. */

s_pu16BufAddr = (uint16_t*)g_au8FrameBuf;

pdma_init();

if (s_i32Channel < 0)

{

/* Allocate a PDMA channel resource. */

s_i32Channel = nu_pdma_channel_allocate(PDMA_MEM);

if (s_i32Channel < 0)

return -1;

}

/* Initial all Lines descriptor-link. */

disp_pdma_dsc_init();

/* Register ISR callback function */

sChnCB.m_eCBType = eCBType_Event;

sChnCB.m_pfnCBHandler = nu_pdma_memfun_cb;

sChnCB.m_pvUserData = (void *)NULL;

nu_pdma_filtering_set(s_i32Channel, NU_PDMA_EVENT_TRANSFER_DONE);

nu_pdma_callback_register(s_i32Channel, &sChnCB);

/* Trigger scatter-gather transferring. */

return nu_pdma_sg_transfer(s_i32Channel, s_head, 0);

}

// Function to deinitialize the EBI sync PDMA

static int disp_sync_pdma_fini(void)

{

if (s_i32Channel >= 0)

{

/* Free allocated PDMA channel resource. */

nu_pdma_channel_free(s_i32Channel);

s_i32Channel = -1;

}

pdma_fini();

return 0;

}

## disp_example組件代碼介紹

In the implementation of disp_example.c, the .incbin assembly directive is used to embed two RGB565 format images directly into the program and store them in FLASH to reduce compilation time. In the disp_example_init function, the callback function for the Blank event (disp_example_blankcb) is first registered. Then, the two RGB565 image data are copied into the image buffer. Since the Cortex-M55 supports Data-Cache, after the copy operation, the function SCB_CleanDCache_by_Addr is called to synchronize the image data to SRAM. In the disp_example_blankcb function, the disp_set_vrambufaddr function is used to alternately update the panel by passing different VRAM addresses, which helps prevent screen tearing. However, because the update rate in the demo example is too fast and causes image ghosting, the condition is set to switch the panel only after receiving the Blank event twice.

#define STR2(x) #x

#define STR(x) STR2(x)

#define INCBIN(name, file) \

__asm__(".section .rodata\n" \

".global incbin_" STR(name) "_start\n" \

".balign 16\n" \

"incbin_" STR(name) "_start:\n" \

".incbin \"" file "\"\n" \

\

".global incbin_" STR(name) "_end\n" \

".balign 1\n" \

"incbin_" STR(name) "_end:\n" \

".byte 0\n" \

); \

extern const __attribute__((aligned(32))) void* incbin_ ## name ## _start; \

extern const void* incbin_ ## name ## _end; \

static uint8_t s_au8FrameBuf[CONFIG_VRAM_TOTAL_ALLOCATED_SIZE] __attribute__((aligned(DCACHE_LINE_SIZE))); // Declare VRAM instance.

INCBIN(image1, PATH_IMAGE1_BIN);

INCBIN(image2, PATH_IMAGE2_BIN);

// Blank event callback function

void disp_example_blankcb(void *p)

{

static uint32_t u32Counter = 0;

/* Toggle different image showing after getting 2 event, */

/* Just for avoid visual persistence ghosting. */

#define DEF_TOGGLE_COND     (u32Counter & 0x10u)

/* Toggle between image1 and image2 display based on u32Counter's value. */

if (DEF_TOGGLE_COND)

{

/* If the condition is true, set VRAM buffer to image2 buffer address. */

disp_set_vrambufaddr((void *)&s_au8FrameBuf[CONFIG_VRAM_BUF_SIZE]);

}

else

{

/* If the condition is false, set VRAM buffer to image1 buffer address. */

disp_set_vrambufaddr((void *)s_au8FrameBuf);

}

// Increment the counter to alternate the display in the next callback

u32Counter++;

}

// Initialize the display example

static int disp_example_init(void)

{

/* Set VRAM buffer address. */

disp_set_vrambufaddr((void *)s_au8FrameBuf);

/* Set blank event callback function. */

disp_set_blankcb(disp_example_blankcb);

/* Copy image1 and image2 pixel data to VRAM buffer. */

memcpy(s_au8FrameBuf, (const uint8_t *)&incbin_image1_start, CONFIG_VRAM_BUF_SIZE);

memcpy(&s_au8FrameBuf[CONFIG_VRAM_BUF_SIZE], (const uint8_t *)&incbin_image2_start, CONFIG_VRAM_BUF_SIZE);

/* Flush all pixel data in DCache to memory. */

SCB_CleanDCache_by_Addr(s_au8FrameBuf, 2 * CONFIG_VRAM_BUF_SIZE);

return 0;

}

## disp驅動參數介紹

The disp.h header file is used to define the synchronization signal pins and timing drive parameters for the synchronous signal LCD panel. These parameters must be configured according to the actual board's pin layout, such as:

- CONFIG_LCD_PANEL_USE_DE_ONLY: When undefined, the system will enable the output of VSYNC_ACT, HSYNC_ACT, and DEACT signals to the panel; otherwise, only the DEACT signal is output.

- CONFIG_DISP_*_ACTIVE_LOW: When set to 0, it indicates the corresponding signal is Active-High; otherwise, it is Active-Low.

- CONFIG_DISP_*_BITIDX: The setting value is the EBI_ADRx pin number plus 1.

- CONFIG_TIMING_*: Used to configure the timing parameters for the synchronization signal LCD panel, which need to be adjusted according to the LCD panel's datasheet.

#define CONFIG_LCD_PANEL_USE_DE_ONLY         /*!< DE-only mode, W/O H/VSync. */

#define CONFIG_DISP_DE_ACTIVE_LOW         0  /*!< Disable DE active low */

#define CONFIG_DISP_VPW_ACTIVE_LOW        1  /*!< Enable VPW active low */

#define CONFIG_DISP_HPW_ACTIVE_LOW        1  /*!< Enable HPW active low */

#define CONFIG_DISP_DE_BITIDX             8  /*!< Implies SET_EBI_ADR7_PH0 */

#define CONFIG_DISP_VSYNC_BITIDX          1  /*!< Implies SET_EBI_ADR0_PH7 */

#define CONFIG_DISP_HSYNC_BITIDX          2  /*!< Implies SET_EBI_ADR1_PH6 */

#define CONFIG_TIMING_HACT              480  /*!< Specify XRES */

#define CONFIG_TIMING_VACT              272  /*!< Specify YRES */

#define CONFIG_TIMING_HBP                30  /*!< Specify HBP (Horizontal Back Porch) */

#define CONFIG_TIMING_HFP                 5  /*!< Specify HFP (Horizontal Front Porch) */

#define CONFIG_TIMING_HPW                41  /*!< Specify HPW (HSYNC plus width) */

#define CONFIG_TIMING_VBP                 2  /*!< Specify VBP (Vertical Back Porch) */

#define CONFIG_TIMING_VFP                27  /*!< Specify VFP (Vertical Front Porch) */

#define CONFIG_TIMING_VPW                10  /*!< Specify VPW (VSYNC width) */

# Software and Hardware Requirements

## Software Requirements

BSP version

M55M1_Series_BSP_CMSIS_V3.01.001。

IDE version

Keil uVision 5.40。

## Hardware Requirements

Circuit components

NuMaker-M55M1 V1.0

EBI16-RGB24 Glue board

Sync-type LCD Panel, Resolution: WQVGA

Schematic diagram of the route

Assemble the NuMaker-M55M1 V1.0, EBI16-RGB24 adapter board, and the synchronous signal LCD panel.

Use an USB Type-C to Type-A cable to connect the NuMaker-M55M1 V1.0 development board to the computer.

Figure 3-1 Pin Connection Diagram

# 目錄資訊

The directory structure is shown in the diagram below.

EC_M55M1_Drive_RGB_LCD_panel_using_EBI_V1.00 - Library; Sample code header and source files. - CMSIS; Cortex® Microcontroller Software Interface Standard (CMSIS) by Arm® Corp.. - Device; CMSIS compliant device header file. - StdDriver; All peripheral driver header and source files. - SampleCode. - ExampleCode.

- Project; Source file of example code.

Figure 4‑1 Directory Structure

# Example Code Execution

Navigate to the ExampleCode section and enter the KEIL folder. Double-click on Drive_RGB_LCD_Panel_using_EBI.uvprojx.

Enter the compilation mode interface:

Compile the code.

Download the code to flash memory.

Run the code.

# Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 2025.04.01 | 1.00 | Initial version。 |